GUIDELINES FOR RESPONSE TO RADIOLOGICAL EMERGENCIES

PURPOSE:

The purpose of this document is to provide guidance for response to hazardous materials incidents involving radioactive materials.

SCOPE:

This procedure applies to hazmat teams in the State of Florida that respond to a hazmat incident involving radioactive material. This procedure will provide guidance to the local hazmat group supervisor, incident commander and supportive staff to meet operational requirements.

PROCEDURE:

1.	Hazardous materials team should request and record all pertinent 
                        information from the dispatching agency. Consider wind direction and 
                        relevant environmental conditions to determine approach route.

2.         Upon arrival, the Hazmat Group Supervisor shall meet with the I.C. and 
                        conduct an incident briefing. Initiate proper documentation for FEMA, 
                        DOH or AHJ records compliance.

3.         Confirm initial radiation readings by first responders, collect any 
                        documentation relevant to the radioactive material and consider any 
                        additional life safety rescues, if applicable.

Personal Radiation Protection (EPA Guidance on Dose Limits for 
                        Emergency personnel) are:

Dose Limit (Rem)                                   Activity

- All

- Protecting valuable property

25                                Life saving or protection of large     
                                    populations

>25                                 Life saving or protection of large  
                                                                 populations only on a voluntary basis to 
                                                                 persons fully aware of the risk involved.

- Validate appropriate isolation distances. Establish a 360 degree perimeter of 2 mR/Hr above background, or less.

5.         Select appropriate PPE to include SCBA.

6.         Determine if any victims/rescuers are contaminated and isolate as 
            necessary. Notify the appropriate hospitals of a radiological incident and 
            report contaminated status of patients, if applicable.

7.         Perform appropriate decontamination, i.e. removal of clothing, dry/wet 
            decon. Monitor for contamination spread.

8.         If containment of radiological material is indicated, perform if appropriate.

9.         Make the appropriate notifications to state and/or local agencies.

10.        Transfer responsibility over to the appropriate agency.

- Ensure proper documentation of radiological exposure information for all rescuers. Ensure response personnel medical monitoring is initiated.

- Approach the scene from upwind, position apparatus 500 feet from the scene or whatever is practical.

- Don full protective gear including SCBA and personal dosimeters.  Designate individual to begin exposure sheet to monitor each person’s exposure rate.  Wear surgical type disposable gloves.

- Obtain as much information about radiological source as possible before approaching it.  Use placards, labels and other sources of information.

- First determine the presence of radiation in the area.  Establish perimeter using banner guard at the 1 mR/hr level for a full 360 degrees.  Use exposure table.

- Make any immediate rescue of victims as required.  Consider all personnel that were in radiation zone to be contaminated until monitoring proves otherwise, if radioactive material was released or spilled.  (Have Medcom advise hospital to prepare for possible radioactive contaminated patients).

- Remember we do not have the proper monitors or training to measure for contamination of individuals.  Establish an isolation area until proper authorities can determine degree of contamination and supervise the decontamination procedure.

- Verify that the proper authorities have been notified through Dispatch.  *See 2.11.13.6

- The maximum exposure for personnel is 25 rems for lifesaving activities.  NO ONE SHOULD ENTER THE RADIATION AREA EXCEPT TO MAKE A RESCUE.

Radioactive I			None

Radioactive II			1 mR/hr

Radioactive III		10 mR/hr

Sole Use Vehicle		At the vehicle surface

Radioactive LSA		200 mR/hr

TO AVIOD UNNECESSARY RADIATION EXPOSURE RELY ON TWO KEY ELEMENTS:

- TIME – The rule is:  Keep contact time with packages short.  Handle packages of radioactive material without delay when moving them.  DO not do time-consuming tasks near packages.